National Cheng Kung University

Department of Electrical Engineering

Introduction to VLSI CAD (Spring 2021)

Lab Session 2

Design and Simulation of Priority Encoder and Ripple Carry Adder

| Name | Student ID |  |  |
| --- | --- | --- | --- |
| 陳旭祺 | E24099059 |  |  |
| Practical Sections: |  | Points | Marks |
| Prob A |  | 25 |  |
| Prob B |  | 25 |  |
| Prob C |  | 30 |  |
| Report |  | 20 |  |
| Notes |  |  |  |

Due Date: 15:00, March 10, 2021 @ moodle

Deliverables

- All Verilog codes including testbenches for each problem should be uploaded.

NOTE: Please DO NOT include source code in the paper report!

- All homework requirements should follow the naming rule in this file hierarchy or you will not get the full credit. (Hint: %sh check.sh)

NOTE: Please DO NOT upload waveforms!

- Important! TA will use the command in Appendix A to check your design under SoC Lab environment, if your code can not be recompiled by TA successfully  using the commands, you will not get the full credit.

- If you upload a dead body, which we cannot even compile, you will get NO credit!

- All Verilog file should get at least 90% Superlint Coverage.

- The name of uploaded file should be Lab2_StudentID.tar, and the student ID must be uppercase. (Ex. Lab2_E24081234.tar)

Fig.1 File hierarchy for Homework submission

Objectives:

Help students get familiar with the CAD tools (NC-Verilog & Verdi) for digital logic design. Please go through the hands-on exercise step-by-step.

Prob A: Design Steps

Understand the function of the target component.

A priority encoder is a device that compresses several inputs into a smaller number of outputs.

Exercise 1-1: Draw the block diagram of a 4-to-2 priority encoder.

Describe in digital logic manner for a simple component. Please fill the blank cells in the truth table and complete the Boolean equation of a 4-to-2 priority encoder.

| Truth Table of a priority encoder |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Inputs |  |  |  | Outputs |  |
| I3 | I2 | I1 | I0 | O1 | O0 |
| 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | x | 0 | 1 |
| 0 | 1 | x | x | 1 | 0 |
| 1 | x | x | x | 1 | 1 |

O0=(I1)(I2)'(I3)'+I3

O1=(I2)(I3)'+I3

Boolean Equation of a priority encoder (the logic of O0, O1)

Derive a gate-level implementation

Draw a Schematic View of a 4-to-2 priority encoder (gate-level diagram)

Implement a 4-to-2 priority encoder based on the gate-level diagram you drawn in 3) by verilog code. (The module name should be encoder)

- You only need to upload your v-code file to moodle, do not paste your code here.

Read the testbench for a 4-to-2 priority encoder you implemented in 4). (The module is named encoder_tb)

- You only need to upload your v-code to moodle, do not paste your code here.

Compile the code you implemented in 4) using NC-Verilog. If there are errors, please go back to 4) to debug your code and re-compile again.

(Hint : The command for compiling is %ncverilog encoder.v)

Simulate your design in 4) with the testbench in 5).

(Hint: The command for simulation is

%ncverilog encoder_tb.v encoder.v +access+r +define+FSDB)

Verify your design by comparing the simulation results with the results you predicted. If the results are not the same, please go back to 4) and revise your code. If the simulation results are correct, please snapshot the simulation result on the terminal and the waveform you dumped and explain your waveform. (Hint : The command to open nWave is %nWave &)

In addition, you should check your coding style, there are no error messages and over 90% coverage with Superlint. Snapshot the result and calculate Superlint coverage. (Hint: The command to open Superlint is %jg -superlint superlint.tcl)

Your simulation result on the terminal

Your waveform

Explanation of your waveform

Priority : I3 > I2 > I1

Superlint screenshot and coverage

Coverage:96% (1 error : I0 is unused)

Prob B: Structral Coding

- An adder is a digital circuit that performs addition of number. Please design a full adder in gate level.

Draw a full adder in gate level.

經Truth Table與K map化簡得 S = A⊕B⊕Cin Cout=AB+A⊕BCin

Design a full adder in Structral coding (The module name should be FullAdder And the file you include should be FullAdder.v)

- You only need to upload your v-code to moodle, do not paste your code here.

Read the testbenche for full adder you implemented in 3). (The module is named FullAdder_tb)

- You only need to upload your v-code to moodle, do not paste your code here.

Compile the code you implemented in 3) using NC-Verilog. If there are errors, please go back to 3) to debug your code and re-compile again. (Hint: The command for compiling is %ncverilog FullAdder.v)

Simulate your design in 3) with the testbench in 4).

(Hint: The command for simulation is % ncverilog FullAdder_tb.v FullAdder.v +access+r +define+FSDB)

Verify your design by comparing the simulation results with the results you predicted. If the results are not the same, please go back to 3) and revise your code. If the simulation results are correct, please snapshot the simulation result on the terminal and the waveform you dumped and explain your waveform. (Hint : The command to open nWave is %nWave &)

In addition, you should check your coding style, there are no error messages and over 90% coverage with Superlint. Snapshot the result and calculate Superlint coverage. (Hint: The command to open Superlint is %jg -superlint superlint.tcl)

Your simulation result on the terminal

Your waveform

Explanation of your waveform

I deem it as {A,B,Cin}={S,Cout} in binary.

Superlint screenshot and coverage

Coverage:100% (No any error or warning)

Prob C: hieratical coding

- Design a 5-bit add/sub ripple carry adder in hierarchical coding.

Your design should be named as ripple_adder.v and module names in the file should be the same as file name (ripple_adder).

Simulate your design in 2) with the following test pattern in sample testbench.

(Hint: The command for compiling is %ncverilog ripple_adder.v)

(Hint: The command for simulation is %ncverilog ripple_adder_tb.v ripple_adder.v +define+FSDB +access+r)

- 2+1

- 7-5

- (-10)+13

- 13+12

- (-12)+(-5)

- (-5)-2

- (-6)-11

- 8-11

- 13-7

- 12-(-6)

Verify your design by comparing the simulation results with the results you predicted. If the results are not the same, please go back to 2) and revise your code. If the simulation results are correct, please snapshot the simulation result on the terminal and the waveform you dumped and explain your waveform. (Hint : The command to open nWave is %nWave &)

In addition, you should check your coding style, there are no error messages and over 90% coverage with Superlint. Snapshot the result and calculate Superlint coverage. (Hint: The command to open Superlint is %jg -superlint superlint.tcl)

Your simulation result on the terminal

Your waveform

Explanation of your waveform

A; A in Bin; B; D in Bin; Add sub; S; Cout; ov_ flag; Ans. 2; 00010; 1; 00001; 0; 00011; 0; 0; 3. 7; 00111; -5; 11011; 0; 00010; 0; 0; 2. -10; 10110; 13; 01101; 0; 00011; 0; 0; 3. 13; 01101; 12; 01100; 0; 11001; 0; 1; 25. -12; 10100; -5; 11011; 0; 01111; 1; 1; -17. -5; 11011; 2; 11110; 1; 11001; 0; 0; -7. -6; 11010; 11; 10101; 1; 01111; 1; 1; -17. 8; 01000; 11; 10101; 1; 11101; 0; 0; -3. 13; 01101; 7; 11001; 1; 00110; 0; 0; 6. 12; 01100; -6; 00110; 1; 10010; 1; 1; 18. If addsub==1,then D=B'+12'complement.Otherwise D=B→ subtractor ifA4==D4==~S4,then ov_flag==1→overflow detector

Superlint screenshot and coverage

Coverage:100% (No any error or warning)

- At last, please write the lesson you learned from Lab 2.

I’ve learn how to design a basic logic unit in gate level or data flow level in Verilog. Furthermore, in Lab3, we use command `include "File_Path/Filename" to call the FullAdder we design in Lab2, which is called hierarchical coding. IMO, it is similar to subfunction in C# and some syntax in Verilog is so closed to C’s that I forgot I’m design hardware instead of software sometimes.

Appendix A : Commands we will use to check your homework

| Problem |  | Command |
| --- | --- | --- |
| ProbA | Compile | %ncverilog encoder.v |
|  | Simulate | %ncverilog encoder_tb.v encoder.v +define+FSDB +access+r |
| ProbB | Compile | %ncverilog FullAdder.v |
|  | Simulate | %ncverilog FullAdder_tb.v FullAdder.v +define+FSDB +access+r |
| ProbC | Compile | % ncverilog ripple_adder.v |
|  | Simulate | %ncverilog ripple_adder_tb.v ripple_adder.v +define+FSDB +access+r |